#include <HX711_ADC.h> // need to install

#include <Wire.h>

#include <LiquidCrystal_I2C.h> // need to install

HX711_ADC LoadCell(6, 7); // parameters: dt pin 6, sck pin 7;

LiquidCrystal_I2C lcd(0x27, 16, 2); // 0x27 is the i2c address might different; you can check with Scanner

const int buzzerPin = 8; // Pin number for the buzzer

const int lowWeightThreshold = 65; // Threshold for low weight in grams

void setup()

{

Serial.begin(9600); // Start serial communication for debugging

LoadCell.begin(); // start connection to HX711

LoadCell.start(2000); // load cells gets 2000ms of time to stabilize

LoadCell.setCalFactor(1000.0); // calibration factor for load cell => dependent on your individual setup

lcd.init();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("     Welcome");

lcd.setCursor(0, 1);

lcd.print("IVBAG Monitoring!");

delay(2000); // Display welcome screen for 2 seconds

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("  SYSTEM CHECK");

delay(2000); // Display system check screen for 2 seconds

lcd.clear();

pinMode(buzzerPin, OUTPUT); // Set the buzzer pin as an output

}

void loop()

{

LoadCell.update(); // retrieves data from the load cell

float weight = LoadCell.getData(); // get output value

// Check if weight is below -2g

if (weight < -2)

{

lcd.setCursor(0, 0); // set cursor to first row

lcd.print("ERROR: NEGATIVE"); // print error message to LCD

lcd.setCursor(0, 1); // set cursor to second row

lcd.print("   WEIGHT!"); // print error message to LCD

delay(2000); // Display error message for 2 seconds

lcd.clear(); // Clear the LCD screen

}

else

{

lcd.setCursor(0, 0); // set cursor to first row

lcd.print("Weight:   g"); // print out to LCD

lcd.setCursor(8, 0); // set cursor to first row, after "Weight:"

lcd.print(weight); // print out the retrieved value to the first row

lcd.setCursor(0, 1); // set cursor to second row

lcd.print("                "); // clear the second row of the LCD

// Check if weight goes below 65g

if (weight < lowWeightThreshold)

{

lcd.setCursor(0, 1); // set cursor to second row

lcd.print("LOW VOLUME!"); // print low weight alert to LCD

tone(buzzerPin, 500, 200); // Set the buzzer to beep beep sound (500Hz tone frequency, 200ms duration)

delay(500); // Delay for 500ms

noTone(buzzerPin); // Turn off the buzzer

}

else

{

lcd.setCursor(0, 1); // set cursor to second row

lcd.print("                "); // clear the second row of the LCD

noTone(buzzerPin); // Turn off the buzzer

}

}

}